HEAVY VEHICLE NATIONAL LAW

Queensland Class 3 Cotton Module Transportation Dimension Exemption Notice 2024 (No.1)

- Purpose

- This notice grants exemptions from height requirements for a class 3 heavy vehicle that is transporting cotton modules.

- This notice also grants an exemption from rear overhang requirements for some chain bed semitrailers, whether transporting cotton modules or not.

- Authorising provision

- This notice is made under the following provision of the Heavy Vehicle National Law (HVNL):

- section 117 – Regulator’s power to exempt category of class 1 or 3 heavy vehicles from compliance with mass or dimension requirement.

- Title

This notice may be cited as the Queensland Class 3 Cotton Module Transportation Dimension Exemption Notice 2024 (No.1).

- Commencement date

This notice commences on 10 February 2024.

- Expiry date

This notice expires on 9 February 2029.

- Definitions

- Unless otherwise stated, words and expressions used in this notice have the same meanings as those in the HVNL and its regulations.

- In this notice, definitions specified in the following notice apply:

- National class 2 B-Double Authorisation Notice, as amended from time to time.

- In this notice:

Guide means the Queensland Class 3 Cotton Module Transportation Dimension Exemption Notice Operator’s Guide published by the National Heavy Vehicle Regulator, and as amended from time to time.

Chain bed semitrailer means a semitrailer equipped with a chain bed designed for the transport of cotton modules.

Cotton module means a packed and compressed cotton bale.

- Application

- This notice applies to a class 3 heavy vehicle that meets all the requirements in this section.

- This notice applies to a heavy vehicle that is:

- a prime mover towing a semitrailer; or

- a B-double.

- This notice applies to a vehicle:

- in the case of a vehicle that has a chain bed semitrailer, whether it is carrying cotton modules or not; or

- in the case of any other vehicle, only when the vehicle is transporting cotton modules.

- This notice applies in Queensland.

- A heavy vehicle to which this section applies and that complies with the conditions of this notice is an eligible vehicle.

- Exemption – Prescribed dimension requirements

- An eligible vehicle is exempt from the following dimension requirements under Schedule 6 of the Heavy Vehicle (Mass Dimension and Loading) National Regulation (MDL Regulation):

- section 5 – Length (rear overhang)

- section 8 – Height

- An exemption in 1) only applies to the extent that specific conditional dimension limits are provided in section 9 of this notice.

- The exemption in (1)(a) only applies to chain bed semitrailer that was constructed or modified before 1 January 2007.

- Condition – Dimension limits

- For a parameter given in Column 1 of Table 1, an eligible vehicle must not exceed the dimension given in Column 2.

Table 1: Maximum dimensions

| Column 1 | Column 2 |
| --- | --- |
| Parameter | Maximum Dimension (m) |
| Length (rear overhang) | 4.2 |
| Height | 4.6 |

- Condition – stated areas or routes

- An eligible vehicle is authorised to access the stated routes and areas specified in this section.

- An eligible vehicle that is a prime mover towing a semitrailer has general access.

- An eligible vehicle that is a compliant B-double, but for the exemption provided in this notice, may operate on networks in the following Gazette notice as though it were a compliant class 2 B-double subject to all conditions of that notice:

National Class 2 B-Double Authorisation Notice

- For the purposes of section 119(1)(a) of the HVNL, an area or route specified in this section is a stated area or route to which this notice applies.

- An eligible vehicle may operate on the routes or areas shown in the following networks:

- “QLD - Cotton Module Transportation Prime Mover and Semi-trailer Network” and

- “QLD - Cotton Module Transportation B-double Network”

- Regardless of any access granted under this section, an eligible vehicle must comply with any conditions or restrictions applied to a given area or route by the Specified Road Manager Network Requirements in the Guide.

- In this section, general access means access to all roads except as restricted by the Specified Road Manager Network Requirements in the Guide or as stated in the map.

- An eligible vehicle operating on an area or route specified in this section must comply with any of the following conditions prescribed for that area or route:

- Road conditions pursuant to section 160 of the HVNL; and

- Travel conditions pursuant to section 161 of the HVNL; and

- Vehicle conditions pursuant to section 162 of the HVNL.

- Condition – Travel restrictions

- The transportation of cotton modules on a vehicle is restricted to the safest direct route from the farm of origin to a cotton gin nominated by the producer of the cotton.

- An enclosed eligible vehicle that has a chain bed semitrailer may only travel unladen:

- to farm sites for the purpose of loading cotton modules, or

- for the purpose of carrying out maintenance on the trailer, or

- for a return trip to the vehicle’s base.

Peter Austin

Director, Policy Implementation

National Heavy Vehicle Regulator