Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2001 (No.1)

The AUSTRALIAN COMMUNICATIONS AUTHORITY issues this Variation under sections 132 and 134 of the Radiocommunications Act 1992.

Dated  20 September 	2001

A.J SHAW                                                                                                                Chair

R HORTON                                                                                                             Deputy Chair

Australian Communications Authority

1	Name of Variation

This Variation is the Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2001 (No.1).

2	Commencement

This Variation commences on gazettal.

3	Amendment of Radiocommunications (Low Interference Potential Devices) Class Licence 2000

Schedule 1 amends the Radiocommunications (Low Interference Potential Devices) Class Licence 2000.

Schedule 1	Amendments

(section 3)

[1]	Section 3, boxed note

omit

(see paragraph 4 (1) (b) and Note 3 after section 4 of this Class Licence).

insert

(see paragraph 4 (1) (b) and Note 1 after section 4 of this Class Licence).

[2]	Section 3, boxed note

omit

ISM applications (see Note 4 after section 4 of this Class Licence).

insert

ISM applications (see Note 2 after section 4 of this Class Licence).

[3]	After section 3

insert

3A	Definitions

device compliance day, for a device, means the most recent of the following days:

(a)	if the device was manufactured in Australia — the day it was manufactured; and

(b)	if the device was manufactured overseas and imported — the day it was imported; and

(c)	if the device was altered or modified in a material respect — the day it was altered or modified.

low interference potential device means a radiocommunications device that complies with the conditions set out in this Class Licence.

Note 1   For the definition of other expressions used in this Class Licence, see the Radiocommunications Act 1992, and the Radiocommunications (Interpretation) Determination 2000.

[4]	Subsection 4 (2), notes 1 and 2

omit

[5]	Subsection 4 (2), note 3

renumber as Note 1

[6]	Subsection 4 (2), note 4

renumber as Note 2

[7]	Section 5, heading

substitute

5	Standards

[8]	Paragraph 5 (1) (a)

omit

the date of commencement of the Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2000 (No.1) 1 (the licence variation);

insert

22 November 2000;

[9]	Paragraph 5 (1) (a), footnote

omit

[10]	Paragraph 5 (1) (b)

omit

the date of commencement of the licence variation;

insert

22 November 2000;

[11]	Subparagraph 5 (1) (c) (ii)

omit

the date of commencement of the licence variation.

insert

22 November 2000.

[12]	Subsection 5 (2)

substitute

(2)	In addition, if the device compliance day for a device authorised under this class licence is on or after the date of commencement of the Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2001 (No.1), the device must comply with any standard applicable to the device on its device compliance day, as in force on that day.

Note 1   Section 5 of the Radiocommunications Act 1992 provides that standard means a standard made under section 162 of that Act.

Note 2   The Australian Communications Authority wishes to make it clear that if a standard mentioned in subsection (2) is amended or replaced by another standard after the device compliance day for the device, the device need not comply with the new or amended standard.

[13]	Schedule 1

substitute

Schedule 1	Transmitters

(section 4)

| Item | Class of transmitter | Permitted operating frequency band (MHz) (lower limit exclusive, upper limit inclusive) | Maximum EIRP | Limitations |
| --- | --- | --- | --- | --- |
| 1 | All transmitters | 0.000–0.014 | 200 µW |  |
| 2 | All transmitters | 0.014–0.01995 | 50 µW |  |
| 3 | All transmitters | 0.02005–0.07 | 7.5 µW |  |
| 4 | All transmitters | 0.07–0.16 | 3 µW |  |
| 5 | All transmitters | 1. 0.16–0.285 2. 0.325–0.415 | 500 nW |  |
| 6 | All transmitters | 3.025–3.155 | 7.5 nW |  |
| 7 | All transmitters | 3.5–3.7 | 30 pW |  |
| 8 | All transmitters | 1. 3.7–3.95 2. 4.438–4.65 | 7.5 nW |  |
| 9 | All transmitters | 13.553–13.567 | 100 mW |  |
| 10 | All transmitters | 24–24.89 | 10 mW |  |
| 11 | All transmitters | 26.957–27.283 | 1 W | 1. Separation of the operating frequency from the centre frequency of any adjacent citizen band radio channel must be at least 5 kHz. 2. The emission bandwidth must not exceed 10 kHz. |
| 12 | All transmitters | 1. 29.7–29.72 2. 30–30.0625 3. 30.3125–31 4. 36.6–37 5. 39–39.7625 6. 40.25–40.66 | 100 mW |  |
| 13 | All transmitters | 40.66–41 | 1 W |  |
| 14 | All transmitters | 54–56 | 2.5 mW |  |
| 15 | All transmitters | 1. 70–70.24375 2. 77.29375–77.49375 3. 150.7875–152.49375 4. 173.29375–174 | 100 mW |  |
| 16 | All transmitters | 1. 225–242 2. 244–267 3. 273–303.95 4. 304.05–328.6 5. 335.4–399.9 | 10 µW |  |
| 17 | All transmitters | 433.05–434.79 | 25 mW |  |
| 18 | All transmitters | 915–928 | 3 mW |  |
| 19 | All transmitters | 2400–2463 | 10 mW |  |
| 20 | All transmitters | 1. 10500–10550 2. 24000–24250 | 100 mW |  |
| 21 | Wireless audio transmitters and auditory assistance transmitters | 88–108 | 10 µW | 1. Emission must be frequency modulated and have a maximum bandwidth of 180 kHz. 2. Transmission in a radio channel must not originate in the licence area of a radio broadcasting station (including a repeater or translator station) operating in the same channel. |
| 22 | Wireless audio transmitters | 1. 174–230 2. 520–820 | 3 mW | 1. The emission must be frequency modulated and have a maximum bandwidth of 330 kHz. 2. Transmission in a TV channel must not originate in the licence area of a TV broadcasting station (including a repeater or translator station) operating in the same channel. |
|  |  |  |  | 3. When in an unused TV channel, but in the licence area of a TV broadcasting station (including a repeater or translator station) operating in an adjacent TV channel, the channel centre frequency of the wireless audio transmitter must be at least 200 kHz above the upper edge of the adjacent TV channel, or 400 kHz below the lower edge of the adjacent TV channel. |
| 23 | Biomedical telemetry transmitters | 174–230 | 10 µW |  |
| 24 | Biomedical telemetry transmitters | 520–668 | 3 mW | Transmission in a TV channel must not originate in the licence area of an analogue TV broadcasting station (including a repeater or translator station) operating in the same channel. |
| 25 | Telecommand or telemetry transmitters | 472.0125–472.1125 | 100 mW |  |
| 26 | Telecommand or telemetry transmitters | 1. 2400–2450 2. 5725–5795 3. 5815–5875 | 1 W |  |
| 27 | Telecommand or telemetry transmitters | 5795–5815 | 2 W |  |
| 28 | Auditory assistance transmitters | 3.155–3.4, with a carrier frequency of: (a) 3.175 MHz; or (b) 3.225 MHz; or (c) 3.275 MHz; or (d) 3.325 MHz. | 60 µW |  |
| 29 | Auditory assistance transmitters | 1. 41–42, with a carrier frequency of: (a) 41.55 MHz; or (b) 41.65 MHz; or (c) 41.75 MHz; or (d) 41.85 MHz; or (e) 41.95 MHz. | 1.3 mW |  |
|  |  | 2. 43–44, with a carrier frequency of: (a) 43.05 MHz; or (b) 43.15 MHz; or (c) 43.25 MHz; or (d) 43.35 MHz; or (e) 43.45 MHz. |  |  |
| 30 | Radiofrequency identification transmitters | 1. 1.77–2.17 2. 2.93–3.58 3. 7.2–10.01 | 100 pW |  |
| 31 | Radiofrequency identification transmitters | 1. 13.553–13.567 2. 918–926 3. 2400–2450 4. 5725–5795 5. 5815–5875 6. 24000–24250 | 1 W |  |
| 32 | Radiofrequency identification transmitters | 5795–5815 | 2 W |  |
| 33 | Alarm transmitters (including security and personal safety transmitters) | 303.60–304.05 | 100 µW |  |
| 34 | Home detention monitoring equipment | 314.075–314.325 | 200 µW | In a 10 second period, a single transmission must not exceed 10 milliseconds. |
| 35 | Radiodetermination transmitters | 24000–24250 | 1 W |  |
| 36 | Radiodetermination transmitters | 60000–61000 | 20 mW |  |
| 37 | Transmitters used for underground communications | 1. 31–32 2. 33–34 3. 35–36 4. 37–38 5. 42–43 6. 44–45 7. 70.24375–74.8 8. 75.2–77.29375 9. 77.49375–84.69375 10. 149.25–149.9 11. 150.05–151.39375 12. 152.49375–156 13. 157.45–160.6 14. 160.975–161.475 15. 162.05–173.29375 16. 403–406 17. 406.1–420 18. 450–500.99375 | 3.5 nW | The maximum EIRP applies at an above-ground opening associated with the underground communications. |
|  |  | 19. 504.99375–510.99375 20. 514.99375–520 |  |  |
| 38 | Transmitters used for underground communications | 1. 0.5265–1.605 2. 87.5–108 | 10 µW | The maximum EIRP applies at an above-ground opening associated with the underground communications. |
| 39 | Aquatic animal tracking transmitters | 48–49 | 10 mW |  |
| 40 | Radiodetermination transmitters operated in radiofrequency-shielded enclosures | 24050–26050 | 75 nW | The maximum EIRP applies outside the shielded enclosure. |
| 41 | Personal alarm transmitters | 27.500–27.510 | 100 µW |  |
| 42 | Transmitters used with personal alarm transmitters operating in the frequency band 27.500–27.510 MHz | 27.500–27.510 | 500 mW | Each transmission must not exceed 4 seconds over a 60 second period. |
| 43 | Alarm transmitters | 344.8–345.2 | 1 mW | The average EIRP must not exceed 100 µW: (a) if the length of a pulse train does not exceed 0.1 second — in the length of one complete pulse train; or (b) if the length of a pulse train exceeds 0.1 second — in the 0.1 second period during which the EIRP is at its maximum value; or (c) if a transmitter operates for more than 0.1 second — in the 0.1 second period during which the EIRP is at its maximum value. |
| 44 | Radio Local Area Network transmitters used indoors | 5150–5350 | 200 mW (averaged over the entire transmission burst) | 1. If the emission bandwidth is 1 MHz or greater, the radiated power spectral density in any 1 MHz is limited to 10 mW per MHz. 2. If the emission bandwidth is less than 1 MHz, the radiated power spectral density in any 4 kHz is limited to 40 µW per 4 kHz. |
| 45 | Radio Local Area Network transmitters | 5725–5825 | 1 W (averaged over the entire transmission burst) | 1. If the emission bandwidth is 1 MHz or greater, the radiated power spectral density in any 1 MHz is limited to 50 mW per MHz. 2. If the emission bandwidth is less than 1 MHz, the radiated power spectral density in any 4 kHz is limited to 200 µW per 4 kHz. |
| 46 | Radiodetermination transmitters | 5725–5875 | 1 mW |  |
| 47 | Radiodetermination transmitters | 76000–77000 | 25 W |  |