Fuel Standard (Automotive Diesel) Determination 2001

as amended

made under section 21 of the

Fuel Quality Standards Act 2000

This compilation was prepared on 5 September 2006
taking into account amendments up to Fuel Standard (Automotive Diesel) Amendment Determination 2005 (No. 1)

The text of any of those amendments not in force on that date is appended in the Notes section

Prepared by the Office of Legislative Drafting and Publishing,
Attorney-General’s Department, Canberra

Contents

1	Name of Determination [see Note 1]	3

2	Commencement	3

2A	Definition	3

3	Fuel standard for diesel	3

4	Testing methods	4

5	Operability standards for diesel	4

Notes 		6

1	Name of Determination [see Note 1]

This Determination is the Fuel Standard (Automotive Diesel) Determination 2001.

2	Commencement

This Determination commences on 1 January 2002.

2A	Definition

In this Determination:

ASTM International means the standards development organisation of that name.

diesel means automotive diesel.

3	Fuel standard for diesel

(1)	Diesel that contains a substance mentioned in the following table must not contain more than the amount mentioned for the substance from the date mentioned for the substance.

| Item | Substance | Amount | Date |  |
| --- | --- | --- | --- | --- |
| 1 | Sulfur | 500 mg/kg | 31 December 2002 |  |
| 2 | Sulfur | 50 mg/kg | 1 January 2006 |  |
| 2A | Sulfur | 10 mg/kg |  | 1 January 2009 |
| 3 | Ash and suspended solids | 100 mg/kg | 1 January 2002 |  |
| 4 | PAH (Polycyclic aromatic hydrocarbons) | 11% mass by mass | 1 January 2006 |  |

(2)	The distillation temperature at which 95% of diesel has been recovered must:

(a)	from 1 July 2005 — not be above 370C; and

(b)	from 1 January 2006 — not be above 360C.

(3)	The cetane index of diesel must be at least 46.

(4)	The density of diesel must be at least 820 kg/m3 and not more than:

(a)	from 1 January 2002 — 860 kg/m3; and

(b)	from 1 January 2006 — 850 kg/m3.

(5)	The viscosity of diesel must be at least 2.0 cSt, and not more than 4.5 cSt, at 40C.

4	Testing methods

(1)	Compliance with the standard set out in section 3 for a substance or parameter is determined by application of the testing method for the substance or parameter in the following table:

| Item | Substance or parameter | Testing method |
| --- | --- | --- |
| 1 | Ash and suspended solids | ASTM D482 |
| 2 | Sulfur | ASTM D5453 |
| 3 | Cetane index | ASTM D4737 Procedure A |
| 4 | Density | ASTM D1298 |
| 5 | Distillation temperature | ASTM D86 |
| 6 | PAH (Polycyclic aromatic hydrocarbons) | IP 391 |
| 7 | Viscosity | ASTM D445 |

(2)	For subsection (1), ASTM followed by an alphanumeric code means the testing method developed by ASTM International under that alphanumeric code.

5	Operability standards for diesel

(1)	To allow the more effective operation of engines, diesel of a kind mentioned in an item of the following table must, in relation to the parameter mentioned in that item, comply with the specification for that parameter mentioned in that item.

(2)	For subsection (1), a parameter of diesel of a kind mentioned in an item of the table complies with the specification for that item only if that is the result after the diesel has been tested, in relation to that parameter, using the ASTM International or Institute of Petroleum (IP) method of testing mentioned in that item.

| Item | Kind of diesel | Parameter | Specification | Referee test method |
| --- | --- | --- | --- | --- |
| 1 | All diesel | Carbon residue (10% distillation residue) | 0.2 mass % maximum | ASTM D4530 |
| 2 | All diesel | Water and sediment | 0.05 vol % maximum | ASTM D2709 |
| 3 | All diesel held by a terminal or refinery for sale or distribution | Conductivity at ambient temperature | 50 per pS/m minimum at ambient temperature | ASTM D2624 |
| 4 | All diesel | Oxidation stability | 25 mg/l maximum | ASTM D2274 |
| 5 | All diesel | Colour | 2 maximum | ASTM D1500 |
| 6 | All diesel | Copper corrosion (3 hours at 50°C) | Class 1 maximum | ASTM D130 |
| 7 | All diesel | Flash point | 61.5°C minimum | ASTM D93 |
| 8 | All diesel | Filter blocking tendency | 2.0 maximum | IP 387 |
| 9 | All diesel containing less than 500 ppm sulfur | Lubricity | 0.460 mm maximum | IP 450 |

Notes to the Fuel Standard (Automotive Diesel) Determination 2001

Note 1

The Fuel Standard (Automotive Diesel) Determination 2001 (in force under section 21 of the Fuel Quality Standards Act 2000) as shown in this compilation is amended as indicated in the Tables below.

Under the Legislative Instruments Act 2003, which came into force on 1 January 2005, it is a requirement for all non-exempt legislative instruments to be registered on the Federal Register of Legislative Instruments.

Table of Instruments

| Title | Date of notification in Gazette or FRLI registration | Date of commencement | Application, saving or transitional provisions |
| --- | --- | --- | --- |
| Fuel Standard (Diesel) Determination 2001 | 22 Aug 2001 (see Gazette 2001, No. GN33) | 1 Jan 2002 |  |
| Fuel Standard (Diesel) Amendment Determination 2001 (No. 1) | 3 Oct 2001 (see Gazette 2001, No. GN39) | 1 Jan 2002 | — |
| Fuel Standard (Diesel) Amendment Determination 2002 (No. 1) | 24 Oct 2002 (see Gazette 2002, No. S396) | 16 Oct 2002 | — |
| Fuel Quality Standards Amendment Determination 2004 (No. 1) | 11 Aug 2004 (see Gazette 2004, No. GN32) | 11 Aug 2004 | — |
| Fuel Standard (Automotive Diesel) Amendment Determination 2005 (No. 1) | 15 June 2005 (see F2005L01417) | 16 June 2005 | — |

Table of Amendments

| ad. = added or inserted am. = amended rep. = repealed rs. = repealed and substituted |  |
| --- | --- |
| Provision affected | How affected |
| S. 1 | rs. 2002 No. 1 |
| S. 2A | ad. 2002 No. 1 |
|  | am. 2005 No. 1 |
| S. 3 | am. 2001 No. 1; 2004 No. 1; 2005 No. 1 |
| S. 4 | ad. 2001 No. 1 |
|  | rs. 2005 No. 1 |
| S. 5 | ad. 2002 No. 1 |
|  | am. 2005 No. 1 |