COMMONWEALTH OF AUSTRALIA	(Civil Aviation Regulations 1998), PART 39 - 105 CIVIL AVIATION SAFETY AUTHORITY

SCHEDULE OF AIRWORTHINESS DIRECTIVES

# AIRWORTHINESS DIRECTIVE

For the reasons set out in the background section, the CASA delegate whose signature appears below issues the following Airworthiness Directive (AD) under subregulation 39.1 (1) of CAR 1998. The AD requires that the action set out in the requirement section (being action that the delegate considers necessary to correct the unsafe condition) be taken in relation to the aircraft or aeronautical product mentioned in the applicability section: (a) in the circumstances mentioned in the requirement section; and (b) in accordance with the instructions set out in the requirement section; and (c) at the time mentioned in the compliance section.

Airbus Industrie A319, A320 and A321 Series Aeroplanes

# AD/A320/123	Ram Air Turbine Actuator	10/2001

Applicability:	A319 and A321 Series aircraft and A320 series aircraft incorporating Airbus Industrie Modification 27189 (Installation of A319/A321 configuration RAT).

Requirement:	Replace the ram air turbine (RAT) actuator in accordance with Airbus Industrie Service Bulletin A320-29-1098.

Note: DGAC AD 2001-236(B) refers.

Compliance:	No later than 31 May 2003.

This Airworthiness Directive becomes effective on 4 October 2001.

Background:	During an operational test of the RAT it did not deploy. Investigation revealed machining chips from manufacture found in the end cap were jamming the actuator.

Eugene Paul Holzapfel

Delegate of the Civil Aviation Safety Authority 14 August 2001

The above AD is notified in the Commonwealth of Australia Gazette on 12 September 2001.

Page 1 of 1