COMMONWEALTH OF AUSTRALIA	(Civil Aviation Regulations 1998), PART 39 - 105 CIVIL AVIATION SAFETY AUTHORITY

SCHEDULE OF AIRWORTHINESS DIRECTIVES

Gulfstream (Grumman American/ American Aviation) AA-1 Series Aeroplanes

# AD/AA-1/14	Rudder Control Bars - Inspection	2/75

Applicability:	All models.

Requirement:	To confirm that rudder control bars P/N 601031-501 are correctly welded inspect visually as follows:

- View both rudder control bars located inside the lower forward fuselage.

- Inspect weld joints at junction of rudder cable attach arm and rudder control bar. Each joint must have a weld bead at four locations i.e. fore and aft bottom and fore and aft top.

- Inspect weld joints at junction of rudder pedal tees and rudder control bar. Each joint must have a weld bead at four locations i.e. fore and aft bottom and fore and aft top.

- If inspection reveals missing welds, replacement or repairs to an approved scheme must be incorporated before further flight.

Compliance:	Before further flight after 21 February 1975.