- MARINE ORDERS

- Part 32

- Cargo Handling Equipment

- Issue 2
(Amendment)

- Order No 2 of 1999

Pursuant to Section 425(1AA) of the Navigation Act 1912, I hereby make this Order amending Marine Orders, Part 32, Issue 2, by omitting pages (i), 13, 14, 101 to 104 and 111, and substituting the attached pages (i), 13, 14, 101 to 104 and 111 to 116, to come into operation on 1 February 1999.
	Clive Davidson
		Chief Executive
	14 January 1999

## Table of Contents

1	Definitions

2	Purpose

3	Application

4	Exemptions & equivalents

5	Transitional

6	Review of decisions

7	Control

8	Penal provisions

9	Person in charge

10	Loading and unloading

11	Access to a ship or compartment

12	Marking of cargo mass

13	Powers and functions of surveyors

14	Safe working load

15	Testing, thorough examination, inspection and certification

16	Register of materials handling equipment and certificates of test

17	Maintenance and repairs

Appendix 1	Protective fencing

Appendix 2	Lighting

Appendix 3	Safe atmosphere

Appendix 4	Personnel cradle

Appendix 5	Test and thorough examination procedures (materials handling equipment)

Appendix 6	Safe use of materials handling equipment

Appendix 7	Cargo spaces

Appendix 8	Motors in cargo spaces

Appendix 9	Requirements for derricks

Appendix 10	Requirements for cranes

Appendix 11	Crane and derrick 
controls and brakes

Appendix 12	Operator's cabin and access

Appendix 13	Mechanical stowing appliances and other vehicles

Appendix l4	Requirements for specialised handling systems

Appendix l5	Requirements for cargo gear

Appendix l6	Unitised cargo

Appendix 17	Ladders

Appendix 18	Signalling

Appendix 19	Marking of mass

Appendix 20	Australian standards: 
guide in determining
SWL

Appendix 21	Actuating force: control equipment

Appendix 22	Approved splices in wire rope

Appendix 23	Forms

Previous issue

Issue 1, Order No 12 of 1986
¾Amendment, Order No. 5 of 1992

Issue 2, Order No 14 of 1997

Note:  Effective measures may include fencing, flagging defective machinery, tagging switches or valves not to be used, cessation of work.

9.2.3  If, in connection with the loading or unloading of a ship:

(a)	a person receives an injury requiring referral to a hospital; or

(b)	a component of materials handling equipment fails in operation, whether or not any person is thereby injured,

the person in charge must submit to a surveyor at the nearest AMSA survey office:

(a)	within four hours of the incident—an initial report in accordance with Form MO-32/5 in Appendix 23; and

(b)	within 72 hours of the incident—a full report in accordance with Form MO-32/6 in Appendix 23.

Note:  Two forms are available from AMSA: Form AMSA 18, which is an Incident Alert form, and AMSA 19, which is an Incident Report form.

# 10  Loading and Unloading

10.1  Person in charge to direct loading and unloading

A person must not, to any extent, load or unload a ship unless that person does so 
under the direction of the person in charge.

10.2  Loading and unloading to comply with this Part

A person must not:

(a)	to any extent, load or unload a ship;

(b)	use or operate any materials handling equipment in connection with the loading or unloading of a ship;

(c)	direct or purport to authorise any other person to load or unload a ship to any extent, or operate materials handling equipment in connection with the loading or unloading of a ship; or

(d)	use or direct or purport to authorise any other person to use in connection with the loading or unloading of a ship, except in the event of an emergency, any means of passage or access,

otherwise than in compliance with or as provided by this Part.

10.3  Use of equipment

10.3.1  Subject to 10.3.2, a person who:

(a)	is under 18 years of age; or

(b)	does not possess the necessary aptitudes and experience,

must not:

(c)	operate any mechanical stowing appliance, crane or winch; or

(d)	give directional signals to a driver of a crane, winch or other mechanical stowing appliance; or

(e)	have responsibility for attending to cargo falls on winch ends or winch drums; or

(f)	perform tasks involving hoisting, lowering or otherwise adjusting derrick gear or other materials handling equipment.

10.3.2  Provision 10.3.1 does not apply to:

(a)	a member of the crew of the ship being loaded or unloaded; or

(b)	a person under training while properly supervised.

10.3.3  A person must not operate power operated hatch covers unless that person is:

(a)	a member of the crew of the ship being loaded or unloaded; or

(b)	a person authorised to do so by the officer in charge of the loading or 
unloading operation.

10.3.4  A person, other than a member of the crew of the ship, must not operate 
ship's side, bow or stern doors, 'tween deck bulkhead doors, ramps, retractable car decks or similar ship equipment unless that person has been specifically authorised by the Master of the ship to do so.

10.4  Protective clothing

A person must not load or unload a ship unless that person wears protective clothing and other protective equipment appropriate for the loading or unloading operation, for the duration of the operation.

Note: It is the responsibilities of employers to define the protective clothing to be worn in any particular circumstances.

# Appendix 20

Australian standards:
Guide in determining safe working load

Note: The Standards are listed by subject matter and full titles are not included.  In some cases the Standard cited may have subsidiary parts or amendments which should be consulted.

# 1  For guidance in determining safe working load

| Blocks, cargo | AS 2089-1993 |
| --- | --- |
| Blocks, chain | AS 1418.2-1997 |
| Chain | AS 2321-1979 |
| Cranes - General | AS 1418.1-1994 |
| - Serial hoists and winches | AS 1418.2-1997 |
| - Bridge, gantry, portal | AS 1418.3-1997 |
| - Mobile | AS 1418.5-1995 |
| Cranes - Safe use of, General requirements | AS 2550.1-1993 |
| Eyebolts | AS 2317-1984 |
| Fibre rope - Care & safe usage | AS 4142.1-1993 |
| - 3 strand hawser laid and 8 strand plaited ropes | AS 4142.2-1993 |
| Hooks | AS 3777-1990 |
| Industrial trucks - General requirements | AS 2359.1-1995 |
| - Operation | AS 2359.2-1985 |
| Lifting components for chain slings | AS 3776-1990 |
| Lifts, escalators and moving walks - General requirements | AS 1735.1-1986 |
| Pallets | AS 4068-1993 |
| Rigging screws and turnbuckles | AS 2319-1984 |
| Shackles | AS 2741-1992 |
| Slings - fibre rope | AS 1380-1998 |
| - synthetic-webbing | AS 1353-1997 |
| - wire rope | AS 1666-1995 |
| - wire coil, flat | AS 1438-1998 |
| - chain - grade T | AS 3775-1990 |
| - roundslings¾synthetic fibre | AS 4497-1997 |
| Swivels | AS 2318-1990 |
| Wire rope | AS 3569-1989 |
| Wire-rope grips for non-lifting applications | AS 2076-1996 |

# 2  Other standards called up in this part

| Acoustics¾hearing protectors | AS 1270-1988 |
| --- | --- |
| Automotive LP Gas Code | AS 1425-1989 |
| Battery operated vehicles | AS 1915-1992 |
| Carbon monoxide analyser equipment | AS 2094-1977 |
| Classification of hazardous areas - Explosive gas atmospheres - Combustible dusts | AS 2430.1-1987 AS 2430.2-1986 |
| Electrical equipment enclosures | AS 1939-1990 |
| Lighting | AS 1680-1990 |
| Portable Fire Extinguishers - General requirements - Foam type - Powder type - Carbon dioxide type - Vaporising liquid type | AS 1841.1-1997 AS 1841.4-1997 AS 1841.5-1997 AS 1841.6-1997 AS 1841.7-1997 |

| Portable Ladders - Metal - Timber | AS 1892.1-1996 AS 1892.2-1992 |
| --- | --- |
| Safe working in a confined space | AS 2865-1995 |
| Vehicles - pollutant test methods | AS 2095.1-1977 |
| Wire rope - Application guide | AS 2759-1985 |

# 3  Other standards that may provide guidance

| Freight containers handling and securing | AS 3711.10-1993 |
| --- | --- |
| Doors clear openings for external single leaf doors | ISO 3796 |

* * * * * *

Appendix 21

Actuating force: control equipment

The maximum actuating force required to operate controls of materials handling equipment is to be no greater than the following:

Finger-operated lever											            	                         10 N (either direction)

Push button                                                      25 N

Hand-operated lever-

console mounted                                     50 N (either direction)

floor mounted                                         400 N

Pedal                                                               600 N

Steering wheel-

manually powered	250 N

power assisted operating	250 N

power assistance not operating	600 N

******

Form MO 32/5

INCIDENT ALERT

| SHIP DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ship's name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IMO number |  |  |  |  |  |  | Flag |  |  |  |  |  |  |  |  |  |
| Call sign |  |  |  |  |  |  | Satcom number |  |  |  |  |  |  |  |  |  |
| Master |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gross tonnage |  |  |  |  |  |  | No. of persons on board |  |  |  |  |  |  |  |  |  |
| Class society |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Propulsion |  |  |  |  |  |  | IOPP certificate date of issue |  |  |  |  |  |  |  |  |  |
| Ship type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Container |  | Tanker |  | Bulk Carrier |  |  |  |  | Tug |  |  | OSV |  | Other |
| Operator's name and address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Responsible Officer (ISM designated person) |  |  |  |  |  |  | Contact number |  |  |  |  |  |  |  |  |  |
| Agents and P&I Club |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INCIDENT DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Voyage From |  |  |  |  |  |  | To |  |  |  |  |  |  |  |  |  |
| Ship's location (eg port, at sea, lat, long) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location on ship where incident occurred |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date & time of incident / / am/pm |  |  |  |  |  |  |  |  |  |  |  | No.of persons involved |  |  |  |  |

| Nature of incident |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Collision |  | MARPOL Ship Defects |  |
|  |  | Grounding |  | Serious Personal Injury |  |
|  |  | Fire |  | Fatality |  |
|  |  | Structural Failure |  | Disappearance |  |
|  |  | Flooding |  | Loss |  |
|  |  | Machinery Breakdown |  | Presumed Lost |  |
|  |  | Cargo Gear |  | Close Quarters Situation |  |
|  |  | Pilotage |  | Births |  |
|  |  | Dangerous Goods |  | Other (specify) |  |
| DESCRIPTION OF INCIDENT/DAMAGE Note: If incident occurred under pilotage, include name of pilot |  |  |  |  |  |
| OTHER RELEVANT INFORMATION |  |  |  |  |  |

Form MO 32/6

INCIDENT REPORT

| SHIP DETAILS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ship's name |  |  |  |  |  |  |  |
| IMO number |  |  |  |  |  |  | Flag |
| Call sign |  |  |  |  |  |  | Satcom number |
| Master |  |  |  |  |  |  |  |
| Gross tonnage |  |  |  |  |  |  | Propulsion |
| Class society |  |  |  |  |  |  |  |
| Operator's name and address |  |  |  |  |  |  |  |
| Agents and P&I Club |  |  |  |  |  |  |  |
| Date & time of incident |  |  |  |  | Ship's location (eg port, at sea, lat, long) |  |  |
| Part 1 - To be completed if reporting under Marine Orders Part 32 |  |  |  |  |  |  |  |
| Person-in-charge |  |  |  |  |  |  |  |
| Employer of person-in-charge |  |  |  |  |  |  |  |
| Incident |  |  |  |  |  |  |  |
|  |  | Injury |  | If injury, complete parts 4 - 8 |  |  |  |
|  |  | Gear failure |  | If gear failure, complete parts 5 - 8 |  |  |  |
|  |  | Dangerous Goods |  | If dangerous goods, complete parts 5 - 8 |  |  |  |

| Part 2 - To be completed if reporting under sections 268, 269 or 417 of the Navigation Act 1912 (other than births) or regulation 4 of the Navigation (Marine Casualty) Regulations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voyage From |  |  |  |  |  |  |  | To |  |  |  |  |  |
| Incident description |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Grounding |  |  |  | Foundering |  |  |  |  |  | Stranding |  |
|  |  | Collision |  |  |  | Capsize |  |  |  |  |  | Flooding |  |
|  |  | Fire |  |  |  | Explosion |  |  |  |  |  | Machinery failure |  |
|  |  | Structural failure |  |  |  | Close Quarters |  |  |  |  |  | Disappearance |  |
|  |  | Loss |  |  |  | Presumed lost |  |  |  |  |  | Injury |  |
|  |  | Death |  |  |  | Dangerous occurrence |  |  |  |  |  |  |  |
|  |  | Other (specify) |  |  |  |  |  |  |  |  |  |  |  |
| Place of incident |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Machinery spaces |  |  |  | Accommodation block |  |  |  |  |  | Galley |  |
|  |  | Deck/cargo spaces |  |  |  | Gangway/pilot ladder |  |  |  |  |  | Wharf |  |
|  |  | Other (specify) |  |  |  |  |  |  |  |  |  |  |  |
| Crew numbers |  |  |  | Passenger numbers |  |  |  |  |  | Cargo |  |  |  |
| Part 3 - To be completed if reporting under section 417 of the Navigation Act 1912 (Births) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Child's name |  |  |  |  |  |  |  |  |  |  |  |  | Gender M/F |
| Date of birth |  |  | Place |  |  |  |  |  |  |  |  |  |  |
| Mother's full name |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Town & country of birth |  |  |  |  |  |  |  |  | Australian resident Yes/No |  |  |  |  |
| Father's full name |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Town & country of birth |  |  |  |  |  |  |  |  | Australian resident Yes/No |  |  |  |  |

| Part 4 - To be completed if reporting under section 107 of the Occupational Health & Safety (Maritime Industry) Act 1993 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name of affected person |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of birth |  |  |  |  |  |  |  |  | Gender M/F |  |  | PIN |  |  |  |  |
| Home address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Australian resident Yes / No |  |  |  | Town & country of birth |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Crew |  |  | Contractor |  |  |  |  |  | Waterside worker |  |  |  |  | Supplier |
|  |  | Watchkeeper |  |  | Other (specify) …………………………………………………………………………….. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | show watch period: from ……………..to ……………….. Time since last rest period ……………………….… |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Logbook entry date |  |  |  | Rank |  |  |  |  |  |  |  |  |  |  |  |  |
| Hours of duty Time on: |  |  |  |  |  | Time off: |  |  |  |  |  | Hours before duty |  |  |  |  |
| Affected area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Head (1) |  |  | Eyes (1) |  |  |  |  |  | Trunk (3) |  |  |  |  | Arms (4) |
|  |  | Hands (4) |  |  | Legs (5) |  |  |  |  |  | Internal (7) |  |  |  |  | Back (3) |
|  |  | Neck (2) |  |  | Fingers (4) |  |  |  |  |  | Feet (5) |  |  |  |  | Toes (5) |
|  |  | Other (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of injury |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Drowning (150) |  |  |  |  |  | Crushing (100) |  |  |  |  |  | Laceration (060/080) |  |  |
|  |  | Burns & scalds (120) |  |  |  |  |  | Hernia (450) |  |  |  |  |  | Fracture (020/010) |  |  |
|  |  | Electric shock (150) |  |  |  |  |  | Amputation (070) |  |  |  |  |  | Foreign Body (110/090) |  |  |
|  |  | Abrasion (090) |  |  |  |  |  | Bruising (100) |  |  |  |  |  | Asphyxia (110/150/140) |  |  |
|  |  | Strain & sprain (040) |  |  |  |  |  | Other (specify) |  |  |  |  |  |  |  |  |
| Result of incident |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Death |  |  |  |  |  | Serious injury |  |  |  |  |  | Minor injury |  |  |
|  |  | Near miss |  |  |  |  |  | Temporary disability |  |  |  |  |  | Partial disability |  |  |
|  |  | Permanent disability |  |  |  |  |  | Disappearance |  |  |  |  |  | Time off work |  |  |
|  |  | Other (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Place of incident |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Machinery spaces |  |  |  |  |  | Accommodation block |  |  |  |  |  | Galley |  |  |
|  |  | Deck/cargo spaces |  |  |  |  |  | Gangway/pilot ladder |  |  |  |  |  | Wharf |  |  |
|  |  | Other (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Incident factors |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Machinery and (mainly) fixed plant (1) |  |  | Materials and substances (6) |  |
|  |  | Mobile plant and transport (2) |  |  | Environmental agencies (7) |  |
|  |  | Powered equipment, tools and appliances (3) |  |  | Animal, human and biological agencies (8) |  |
|  |  | Non-powered hand tools, appliances and other equipment (4) |  |  |  |  |
|  |  | Chemicals and chemical products (5) |  |  | Other and unspecified agencies (9) |  |
| Cause of injury/illness |  |  |  |  |  |  |
| Date left ship |  |  | Expected period of incapacity |  |  |  |
| Treatment given on board ship |  |  |  |  |  |  |
| Part 5 - Explanation and Description State Cause and give names and addresses of any witnesses |  |  |  |  |  |  |
| Part 6 - Incident narrative |  |  |  |  |  |  |
| Part 7 - Action taken to prevent similar occurrences |  |  |  |  |  |  |
| Part 8 - Details of person completing report |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |
| Position |  |  | Contact number |  |  |  |
| Signature |  |  |  |  |  | Date / / |