Radiocommunications (Digital Radio Channels — NSW/ACT) Plan 2007

Radiocommunications Act 1992

The AUSTRALIAN COMMUNICATIONS AND MEDIA AUTHORITY makes this Plan under subsection 44A (1) of the Radiocommunications Act 1992.

Dated 6 December 2007

Chris Chapman
[signed]
Member

Lyn Maddock
[signed]
Member

Australian Communications and Media Authority

1	Name of Plan

This Plan is the Radiocommunications (Digital Radio Channels — NSW/ACT) Plan .

2	Commencement

This Plan commences on the day after it is registered.

3	Definitions

In this Plan:

Act means the Radiocommunications Act 1992.

category means a category of digital radio multiplex transmitter licence described in section 5 of the Act.

DAB means digital audio broadcasting.

designated BSA radio area means:

(a)	the BSA licence area (within the meaning of the Broadcasting Services Act 1992) of a commercial radio broadcasting licence; or

(b)	the BSA licence area of a community radio broadcasting licence, where that BSA licence area is the same (or is deemed to be the same) as the BSA licence area of a commercial radio broadcasting licence.

frequency block means a frequency channel of 1.536 MHz bandwidth.

maximum antenna height means the maximum permitted height above ground level of the electrical centre of an antenna.

technical planning guidelines means the guidelines made under section 33 of the Broadcasting Services Act 1992, as in force from time to time.

4	Digital radio channel plans

(1)	For subsection 44A (1) of the Act, each Schedule is a digital radio channel plan for the designated BSA radio area specified in that Schedule.

(2)	The digital radio channel plan specified in a Schedule:

(a)	allots a frequency block or blocks in relation to the designated BSA radio area for use by digital radio multiplex transmitter licensees; and

(b)	reserves a frequency block for a category 3 digital radio multiplex transmitter licence for the designated BSA radio area to be issued in accordance with subsection 102E (2) of the Act; and

(c)	determines which of the following types of licences, or which combination of the following types of licences, are to be issued for the designated BSA radio area:

(i)	category 1 digital radio multiplex transmitter licence;

(ii)	category 2 digital radio multiplex transmitter licence; and

(d)	if a particular type of licence mentioned in paragraph (c) is to be issued for the designated BSA radio area — determines whether:

(i)	a single licence of that type is to be issued for the designated BSA radio area; or

(ii)	2 or more licences of that type are to be issued for the designated BSA radio area; and

(e)	determines technical specifications of multiplex transmitters operated under digital radio multiplex transmitter licences for the designated BSA radio area.

Note   The relevant technical specifications are set out in each Schedule as “attachments”.

Schedule 1	Sydney RA1

(subsection 4 (1))

Designated BSA radio area

Sydney RA1

Table 1	Frequency channels

| Column 1 Multiplex Name | Column 2 Frequency block | Column 3 Reserved frequency block | Column 4 Centre frequency (MHz) | Column 5 Category | Column 6 Technical Specification Number | Column 7 Technical Specifications (Attachment number) |
| --- | --- | --- | --- | --- | --- | --- |
| Sydney 1 | 9A | No | 202.928 | 1 | TS1132485 | 1.1 |
| Sydney 2 | 9B | No | 204.640 | 1 | TS1132486 | 1.2 |
| Sydney 3 | 9C | Yes | 206.352 | 3 | TS1132487 | 1.3 |

Table 2	Number of licences to be issued

| Column 1 Licence Category | Column 2 Number of licences |
| --- | --- |
| Category 1 | 2 |
| Category 2 | 0 |
| Category 3 | 1 |

Attachment 1.1	Multiplex Sydney 1

| Column 1 Technical specification | Column 2 Details |
| --- | --- |
| Category | 1 |
| General Area Served | Sydney |
| Mode | DAB |
| Specification Number | TS1132485 |
| Transmitter Site |  |
| Nominal Location | TCN 9 Television Tower Artarmon Road WILLOUGHBY |
| Australian Map Grid Reference 1966 | Zone Easting Northing 56 332920 6257070 |
| Site Tolerance | Refer to technical planning guidelines |
| Emission |  |
| Frequency Band | VHF |
| Centre Frequency | 202.928 MHz (Frequency Block 9A) |
| Polarisation | Vertical |
| Maximum antenna height | 223 m |
| Output Radiation Pattern |  |
| Bearing or Sector (Clockwise direction) | Maximum ERP |
| 0°T – 360°T | 12.5 kW |
| Additional technical specification |  |
| The radiated signal characteristics of the transmitter must comply with the spectrum mask for VHF transmitters in critical areas for adjacent channel interference as specified in clause 15.4 of standard ETSI EN 300 401 V1.4.1, issued by the European Telecommunications Standard Institute. |  |

Attachment 1.2	Multiplex Sydney 2

| Column 1 Technical specification | Column 2 Details |
| --- | --- |
| Category | 1 |
| General Area Served | Sydney |
| Mode | DAB |
| Specification Number | TS1132486 |
| Transmitter Site |  |
| Nominal Location | TCN 9 Television Tower Artarmon Road WILLOUGHBY |
| Australian Map Grid Reference 1966 | Zone Easting Northing 56 332920 6257070 |
| Site Tolerance | Refer to technical planning guidelines |
| Emission |  |
| Frequency Band | VHF |
| Centre Frequency | 204.640 MHz (Frequency Block 9B) |
| Polarisation | Vertical |
| Maximum antenna height | 223 m |
| Output Radiation Pattern |  |
| Bearing or Sector (Clockwise direction) | Maximum ERP |
| 0°T – 360°T | 12.5 kW |

Attachment 1.3	Multiplex Sydney 3

| Column 1 Technical specification | Column 2 Details |
| --- | --- |
| Category | 3 |
| General Area Served | Sydney |
| Mode | DAB |
| Specification Number | TS1132487 |
| Transmitter Site |  |
| Nominal Location | TCN 9 Television Tower Artarmon Road WILLOUGHBY |
| Australian Map Grid Reference 1966 | Zone Easting Northing 56 332920 6257070 |
| Site Tolerance | Refer to technical planning guidelines |
| Emission |  |
| Frequency Band | VHF |
| Centre Frequency | 206.352 MHz (Frequency Block 9C) |
| Polarisation | Vertical |
| Maximum antenna height | 223 m |
| Output Radiation Pattern |  |
| Bearing or Sector (Clockwise direction) | Maximum ERP |
| 0°T – 360°T | 12.5 kW |