Fuel Standard (Biodiesel) Amendment Determination  2009 (No. 1)1

Fuel Quality Standards Act 2000

I, PETER ROBERT GARRETT, Minister for the Environment, Heritage and the Arts, make this Determination under section 21 of the Fuel Quality Standards Act 2000.

Dated 6 February 2009

PETER ROBERT GARRETT

Minister for the Environment, Heritage and the Arts

1	Name of Determination

This Determination is the Fuel Standard (Biodiesel) Amendment Determination  2009 (No. 1).

2	Commencement

This Determination commences on the day after it is registered.

3	Amendment of Fuel Standard (Biodiesel) Determination 2003

Schedule 1 amends the Fuel Standard (Biodiesel) Determination 2003.

Schedule 1	Amendments

(section 3)

[1]	Section 3, definition of Energy Institute

omit

[2]	Subsection 4 (1), table, item 4

omit

| 4 | Carbon residue — 10% distillation residue; or | 0.30% mass | 18 September 2003 |
| --- | --- | --- | --- |
|  | Carbon residue — 100% distillation sample | 0.050% mass | 18 September 2003 |

insert

| 4 | Carbon residue — 10% distillation residue | 0.30% mass | 18 September 2003 |
| --- | --- | --- | --- |

[3]	Subsection 4 (2), table, item 9

substitute

| 9 | Cetane number; or Derived cetane number | 51.0 (min) 51.0 (min) | 18 September 2005 1 March 2009 |
| --- | --- | --- | --- |

[4]	Section 5

substitute

5	Testing methods

(1)	Compliance with the standard set out in section 4 for the substance or property is determined by the testing method, as existing on the commencement of the Fuel Standard (Biodiesel) Amendment Determination 2009 (No. 1), for the substance or property in the following table.

| Item | Substance or property | Testing method |
| --- | --- | --- |
| 1 | Acid value | ASTM D664 |
| 2 | Methanol | EN 14110 |
| 3 | Carbon residue — 10% distillation residue | ASTM D4530 |
| 4 | Cetane number | ASTM D613 |
| 5 | Derived cetane number | ASTM D6890 |
| 6 | Contamination (total) | EN 12662 |
| 7 | Copper strip corrosion | (a) for specification Class 1 (max) — EN ISO 2160 or ASTM D130 (b) for specification No. 3 (max) — ASTM D130 |
| 8 | Density | ASTM D1298 |
| 9 | Distillation T90 | ASTM D1160 |
| 10 | Ester content | EN 14103 |
| 11 | Flashpoint | ASTM D93 |
| 12 | Glycerol (free) | ASTM D6584 |
| 13 | Glycerol (total) | ASTM D6584 |
| 14 | Metals — Group I (Na, K) | EN 14538 |
| 15 | Metals — Group II (Ca, Mg) | EN 14538 |
| 16 | Oxidation stability | prEN 15751 or EN 14112 |
| 17 | Phosphorus | EN 14107 |
| 18 | Sulfur | ASTM D5453 |
| 19 | Sulfated ash | ASTM D874 |
| 20 | Viscosity | ASTM D445 |
| 21 | Water and sediment | ASTM D2709 |

(2)	For subsection (1):

(a)	ASTM followed by an alphanumeric code means the testing method developed by ASTM International under the alphanumeric code; and

(b)	prEN, EN and EN ISO followed by a number means the testing method developed by the European Committee for Standardization (CEN) under the code and number.

(3)	For item 10 of the table in subsection (1), if biodiesel contains C-17-methyl esters, the ester content may be measured by using the modified procedure set out in S. Schober, I. Seidl and M. Mittelbach, ‘Ester content evaluation in biodiesel from animal fats and lauric oils’, European Journal of Lipid Science and Technology, vol 108, issue 4, 2006, pp 309–314.

Note

1.	All legislative instruments and compilations are registered on the Federal Register of Legislative Instruments kept under the Legislative Instruments Act 2003. See http://www.frli.gov.au.