Vehicle Standard (Australian Design Rule 62/02 – Mechanical Connections Between Vehicles) 2007 Amendment 1

I, ANTHONY NORMAN ALBANESE, Minister for Infrastructure, Transport, Regional Development and Local Government, determine this vehicle standard under section 7 of the Motor Vehicle Standards Act 1989.

Dated		28 	January  2010

[Signed]

Anthony Norman Albanese

Minister for Infrastructure, Transport, Regional Development and Local Government

CONTENTS

1.	legislative provisions	3

2.	amendment of vehicle standard	3

Schedule 1	4

- legislative provisions

  - Name of Legislative Instrument

    - This instrument is the Vehicle Standard (Australian Design Rule 62/02 – Mechanical Connections Between Vehicles) 2007 Amendment 1.

  - Commencement

    - This instrument commences on the day after it is registered.

- amendment of vehicle standard

  - The changes specified in Schedule 1 amend Vehicle Standard (Australian Design Rule 62/02 – Mechanical Connections Between Vehicles) 2007.

Schedule 1

- Clause 13.3.1.1.2 amend to read “For vehicles towing trailers over 23.5 tonnes ‘ATM’, and ‘Converter Dollies’ the lesser of 350 kN; or”

- Clause 13.4.2.1 amend to read “Longitudinal tension (N) = 9.81 x ‘ATM’ (kg); and”

- Clause 13.4.2.2 amend to read “vertical tension (N) = 0.5 x 9.81 x ‘ATM’ (kg).”

- Clause 14.2.1.1.2 amend to read “For trailers over 23.5 tonnes ‘ATM’ and all ‘Converter Dollies’, the lesser of 350 kN or 2.25 x ‘Coupling’ ‘D-value’ (kN) for the ‘Coupling’ ‘D-value’ at which the ‘Drawbar’ is rated.”

- Clause 14.3.1.3 amend to read “For trailers over 3.5 tonnes ‘ATM’, the safety chain must be made from steel of a minimum 800 MPa breaking stress, conforming to the mechanical properties of Grade T chain as specified in AS 2321 - 2001 Short Link Chain for Lifting Purposes and must be of a size such that the minimum breaking load exceeds the ‘ATM’, or a size as shown in Table 1.”

- Following clause 14.3.1.3 insert table as shown:

| ‘Aggregate Trailer Mass’ (tonnes) | Chain Size (millimetres) | Minimum Chain Breaking Load (tonnes) |
| --- | --- | --- |
| Over 3 .5 and up to 5.0 | 6 | 5.1 |
| Over 5.0 and up to 8.0 | 8 | 8.2 |
| Over 8.0 and up to 12.5 | 10 | 12.8 |
| Over 12.5 and up to 21.5 | 13 | 21.7 |
| Over 21.5 and up to 32.5 | 16 | 32.8 |
| Over 32.5 | 19 | 46.5 |

Table 1: Safety Chain Size Selection

- Clause 14.4.2.1 amend to read “Longitudinal tension (N) = 9.81 x ‘ATM’ (kg); and”

- Clause 14.4.2.2 amend to read “vertical tension (N) = 0.5 x 9.81 x ‘ATM’ (kg).”

- Insert new clause 5.2 “The ‘Tow Coupling Overhang’ must not exceed, in the case of an NC vehicle designed for use in ‘Road Trains’, 2.7m.”