Radiocommunications (Low Interference Potential Devices) Class Licence Variation Notice 2010 (No. 1)1

Radiocommunications Act 1992

The AUSTRALIAN COMMUNICATIONS AND MEDIA AUTHORITY makes this Notice under section 134 of the Radiocommunications Act 1992.

Dated 1 September 2010

Chris Chapman 
[signed] 
Member

Nerida O’Loughlin 
[signed] 
Member/General Manager

Australian Communications and Media Authority

1	Name of Notice

This Notice is the Radiocommunications (Low Interference Potential Devices) Class Licence Variation Notice 2010 (No. 1).

2	Commencement

This Notice commences on the later of:

(a)	the day after it is registered; and

(b)	the day on which it is published in the Gazette.

3	Amendment of Radiocommunications (Low Interference Potential Devices) Class Licence 2000

Schedule 1 varies the Radiocommunications (Low Interference Potential Devices) Class Licence 2000.

Schedule 1	Variations

(section 3)

[1]	Section 3A, definition of maximum EIRP

substitute

maximum EIRP means the largest amount of peak equivalent isotropically radiated power that is radiated in any direction from any of the following:

(a)	an antenna that is an integral part of the transmitter;

(b)	an antenna that is connected to the transmitter;

(c)	the surface of a specified enclosure containing the antenna;

(d)	for an item in Schedule 1 that mentions an opening and an underground environment — the opening to the underground environment.

[2]	Schedule 1, item 20

substitute

| 20 | All transmitters | 1. 10500–10550 2. 24000–24250 3. 61000–61500 | 100 mW |  |
| --- | --- | --- | --- | --- |

[3]	Schedule 1, after item 25

insert

| 25A | Telecommand or telemetry transmitters | 1. 0.07–0.119 2. 0.135–0.160 | 10 mW |  |
| --- | --- | --- | --- | --- |
| 25B | Telecommand or telemetry transmitters | 0.119–0.135 | 1.5 W |  |

[4]	Schedule 1, item 51A

substitute

| 51A | Data communications transmitters used indoors | 57000–66000 | 20 W | 1. The average power density of any emission must not exceed 9 uW/cm2 at a distance of 3m. 2. The peak power density of any emission must not exceed 18 uW/cm2 at a distance of 3m. 3. Spurious emissions outside the band must be less than -30dBm/MHz. |
| --- | --- | --- | --- | --- |

[5]	Schedule 1, after item 59

insert

| 60 | Radiodeterminationtransmitters | 77000–81000 | 315 W | 1. Must meet the requirements of ETSI EN 302 264-1 as it applies from time to time. |
| --- | --- | --- | --- | --- |
|  |  |  |  | 2. Must not be operated within a nominated distance of a specified Australian radio-astronomy site. |

Note

1.	All legislative instruments and compilations are registered on the Federal Register of Legislative Instruments kept under the Legislative Instruments Act 2003. See http://www.frli.gov.au.