I, JOHN FRANCIS McCORMICK, Director of Aviation Safety, on behalf of CASA, make this instrument under regulation 66.015 of the Civil Aviation Safety Regulations 1998.

[Signed John F. McCormick]

John F. McCormick
Director of Aviation Safety

21 June 2011

- Manual of Standards Part 66 Amendment Instrument (No. 2) 2011

- 1	Name of instrument

This instrument is the Manual of Standards Part 66 Amendment Instrument (No. 2) 2011.

- 2	Commencement

This instrument commences on 27 June 2011 immediately after the commencement of the Manual of Standards Part 66 Instrument 2011.

3	Amendment of Manual of Standards Part 66 Instrument 2011

The Manual of Standards Part 66 Instrument 2011 is amended as set out in Schedule 1.

Schedule 1	Amendments

[1]	Section 1

substitute

66.1A	Name of instrument

This instrument is the Part 66 Manual of Standards (MOS).

- [2]	Section 2

renumber as section 66.1B

- [3]	Section 3

repeal

- [4]	Schedule 1, heading

omit

- [5]	Schedule 1

omit

- Continuing Airworthiness Aircraft Engineer Licences and Ratings

- [6]	Paragraph 66.5 (b), after the definition of simple test

insert

sub system means a system which, while capable of functioning on its own, is part of a larger system and includes, for this MOS:

1.	the electrical sub system comprised of electrical parts, appliances and motors, within mechanical, powerplant and structural systems; or

2.	the instrument sub system comprised of avionic systems within mechanical, powerplant and structural systems.

- [7]	Sub-subparagraph 66.A.20 (a) 1. (ii)

omit

Appendix V of MOS Part 145

insert

Appendix II of the Part 145 MOS

- [8]	Subparagraph 66.A.20 (a) 2

omit

that subcategory

insert

line

- [9]	Sub-subparagraph 66.A.20 (a) 2. (iii)

omit

Appendix V of MOS Part 145

insert

Appendix II of the Part 145 MOS

- [10]	Subparagraph 66.A.20 (a) 2, after sub-subparagraph 66.A.20 (a) 2. (iii)

insert

and (iv)	the aircraft being maintained is covered by the subcategory of licence held;

- [11]	Subparagraph 66.A.20 (a) 3

omit

include supervision of maintenance

insert

include:

(i)	supervision of maintenance; or

(ii)	for a subcategory A1 licence, maintenance of a propeller-driven aeroplane unless the subcategory A1 licence holder:

(A)	holds a B1.1 subcategory endorsement that permits the holder to perform maintenance certification on the propellor system of a propeller-driven aeroplane; or

(B)	is positively endorsed in the subcategory A1 to provide such maintenance.

- [12]	Sub-sub-subparagraph 66.A.20 (a) 4. (ii) (C)

omit

MOS Part 145

insert

the Part 145 MOS

- [13]	Subparagraph 66.A.20 (a) 5

omit the first mention of

maintenance

insert

aircraft

- [14]	Subparagraph 66.A.20 (a) 5

after the second mention of

licence

insert

, after maintenance of the aircraft,

- [15]	Sub-sub-subparagraph 66.A.20 (a) 6. (ii) (B)

omit

subsystem

insert

sub system

- [16]	Subparagraph 66.A.20 (a) 7

omit the first mention of

maintenance

- [17]	Sub-subparagraph 66.A.20 (a) 8. (i)

omit

particular type of

- [18]	Sub-subparagraph 66.A.20 (a) 8. (ii)

omit

particular type of

- [19]	Subparagraph 66.A.20 (b) 1

omit

the licence

insert

any of the licences

- [20]	Table 1, including the heading

substitute

Aircraft systems, designations and conditions for Category B1 and Category B2 licences

| Aircraft system (and ATA chapter reference) | Designation of system | Conditions or limitations |
| --- | --- | --- |
| Pressurisation, air-conditioning and equipment cooling systems (ATA21) | Mechanical (B1) | For a Category B2 licence, pressurisation control systems, if endorsed on the licence. |
| Autopilot (ATA22) | Avionic (B2) |  |
| Communications (ATA23), including ELT and underwater locating beacon (ATA25-60) | Avionic (B2) |  |
| Generator and/or constant speed drive/IDG systems (ATA24) | Electrical (B1/B2) and Powerplant (B1) |  |
| Electrical power supply systems, including a ram air turbine, if electrical (ATA24) | Electrical (B1 and B2) |  |
| Equipment, furnishings and emergency equipment (ATA25) | Mechanical (B1) |  |
| Fire, smoke, overheat detecting and extinguishing systems (ATA26) | Mechanical (B1) |  |
| Flight control systems (ATA27) | Mechanical (B1) |  |
| Flight control systems – system operation – fly-by-wire (ATA27) | Avionic (B2) | For a Category B2 licence — limited to the avionic subsystem of the flight control system. |
| Fuel systems (ATA28) | Mechanical (B1) |  |
| Hydraulic power systems, including ram air turbine (ATA29) | Mechanical (B1) |  |
| Ice and rain protection systems (ATA30) | Mechanical (B1) |  |
| Ice and rain protection systems (ATA30-20) | Mechanical and Powerplant (B1) | Powerplant — for powerplant cowling anti-icing. |
| Indicating and recording systems (ATA31) | Avionic (B2) |  |
| Landing gear (ATA32) | Mechanical (B1) |  |
| Wheels and brakes (ATA32-40) | Mechanical (B1) |  |
| Lighting (ATA33) | Electrical (B1 and B2) |  |
| Navigation systems: - General - Radio interface - ACARS, SELCAL, INS/IRS - Compass - Flight manage-ment system - Doppler systems (ATA34) | Avionic (B2) | For a Category B1 licence — compass swings, if endorsed on the licence. |
| Oxygen system (ATA35) | Mechanical (B1) | For a Category B2 licence, if endorsed on the licence. |
| Pneumatic system (ATA36) | Mechanical (B1) |  |
| Vacuum (ATA37) | Mechanical (B1) |  |
| Waste water (ATA38) | Mechanical (B1) |  |
| Cabin intercom data and network systems (ATA42) | Avionic (B2) |  |
| Cabin systems (ATA44) | Avionic (B2) |  |
| Central maintenance system (ATA45) | Avionic (B2) |  |
| Information systems - ATIMS - Network server (ATA46) | Avionic (B2) |  |
| Nitrogen generation system or inert gas system (ATA47) | Mechanical (B1) |  |
| APU (ATA49) | Powerplant (B1) |  |
| Cargo and accessory compartments (ATA50) | Mechanical (B1) |  |
| Structures — General (ATA51) | Structural (B1) | Structures — general, but excluding wooden structures and fabric surfaces unless: (a) for wooden structures — the holder has obtained the relevant optional units of competency mentioned in section 66.A.25 of this MOS; or (b) for fabric surfaces — the holder has obtained the relevant optional units of competency mentioned in section 66.A.25 of this MOS. Note These optional units of competency are marked Z in Appendix IV. |
| Doors (ATA52) | Structural (B1) |  |
| Fuselage (ATA53) | Structural (B1) |  |
| Nacelles and pylons (ATA54) | Structural (B1) |  |
| Stabilisers (ATA55) | Structural (B1) |  |
| Windows (ATA56) | Structural (B1) |  |
| Wings (ATA57) | Structural (B1) |  |
| Propeller — rotor (ATA60) | Mechanical (B1) |  |
| Propeller — propulsion (ATA61) | Powerplant (B1) | For a Category B1 licence, only if the holder has obtained the relevant optional units of competency mentioned in section 66.A.25 of this MOS. Note These optional units of competency are marked P in Appendix IV. |
| Rotor (ATA62) | Mechanical (B1) |  |
| Rotor drives (ATA63) | Mechanical (B1) |  |
| Tail rotor (ATA64) | Mechanical (B1) |  |
| Tail rotor drive (ATA65) | Mechanical (B1) |  |
| Folding blades and pylon (ATA66) | Mechanical (B1) |  |
| Rotor flight control (ATA67) | Mechanical (B1) |  |
| Powerplant (ATA71) | Powerplant (B1) |  |
| Engine turbine/ turbo-prop and fans (ATA72) | Powerplant (B1) |  |
| Engine-mounted accessories: gear boxes, gears, pumps and attached engine-mounted and driven components (ATA72-60) | Powerplant (B1) |  |
| Engine fuel and control — carbura-tion/injection system (ATA73) | Powerplant (B1) |  |
| FADEC (ATA73A) | Avionic (B2) |  |
| Ignition system (ATA74) | Powerplant (B1) |  |
| Air systems and control (ATA75) | Powerplant (B1) |  |
| Engine control system (ATA76) | Powerplant (B1) |  |
| Engine indicating system (ATA77) | Powerplant (B1) and Avionic (B2) |  |
| Exhaust — thrust reverser (ATA78) | Powerplant (B1) |  |
| Lubrication system (ATA79) | Powerplant (B1) |  |
| Starting system (ATA80) | Powerplant (B1) |  |
| Supercharging system (ATA81) | Powerplant (B1) |  |
| Power augmentation (ATA82) | Powerplant (B1) |  |
| Accessory drives (ATA83) | Powerplant (B1) |  |
| Transitional privileges Despite Table 1 and the exclusions annotated on a licence issued under Part 66 of CASR 1998, and to be without doubt, a person who, in accordance with regulation 202.341, 202.342, 202.343 or 202.344, held, or was taken to be entitled to the issue of, an aircraft maintenance engineer licence by: 1. previously holding an engine category Group 1 or 2 rating, or an airframe category Group 1, 2 or 19 rating, may perform maintenance certifications and issue certificates of release to service in relation to any of the following kinds of maintenance and to which that licence and rating applied: (a) for a Category B1 licence, on aircraft: - fitted with a single generator; and - approved only for V.F.R. operations; all electrical maintenance. (b) for a Category B1 licence, on aircraft approved only for V.F.R. operations — for aircraft general instruments (excluding, RMI, inertial navigation, multi-axis autopilots), all instrument system maintenance. 2. previously holding an engine or airframe category rated licence, may perform maintenance certifications and issue certificates of release to service in relation to any of the following kinds of maintenance and to which that licence and rating applied: for a category B1 licence, for structural, powerplant, mechanical, electrical and avionic systems: - daily, periodic or manufacturers’ equivalent inspection; and - check of the condition of security of attachment of wiring, plumbing, parts and appliances; and - maintenance of instrument, or electrical, parts and appliances forming part of the airframe system, where the maintenance: - – is limited to external mechanical adjustments to facilitate correct operation of airframe systems; or - – is limited to replacement of instrument, or electrical, parts and appliances, connected by simple twist or terminal connectors; and - – excludes instrument or electrical parts and appliances, where maintenance involves functional tests and adjustments requiring the use of external specialised test equipment. |  |  |

- [21]	Subparagraph 66.A.25 (b) 3

omit

of

insert

to

- [22]	Paragraph 66.A.25 (c)

after

subcategory

insert

A1 or

- [23]	Paragraph 66.A.25 (d)

after

subcategory

insert

A2 or

- [24]	Note after paragraph 66.A.45 (ba)

omit

the CASR 66.010

insert

regulation 66.010 of CASR 1998

- [25]	Note after paragraph 66.A.45 (bb)

omit

the CASR 66.010

insert

regulation 66.010 of CASR 1998

- [26]	Paragraph 66.A.45 (h)

omit

approved under section 145.A.37

insert

, in accordance with section 145.A.37 of the Part 145 MOS,

- [27]	Subparagraph 66.A.45 (h) (ii)

omit

if the subcategory, category and aircraft for the aircraft are

insert

for a category or subcategory of licence if the aircraft, system or subsystem is

- [28]	Appendix 1, Part 3, Module 7, 7.7

omit each mention of

2

insert

3

- [29]	Appendix I, Part 3, Module 10, 10.1

omit

CASR Part 42, Part 66, Part 145, Part 147

insert

Parts 42, 66, 145 and 147 of CASR 1998

- [30]	Appendix I, Part 3, Module 10, 10.2

omit

CASR Part 66

insert

Part 66 of CASR 1998

- [31]	Appendix I, Part 3, Module 10, 10.3

omit

CASR Part 145

insert

Part 145 of CASR 1998

- [32]	Appendix I, Part 3, Module 10, 10.6

omit

CASR Part 42

insert

Part 42 of CASR 1998

- [33]	Appendix II, section 1, subclause 1.11, first sentence

omit

5

insert

10

- [34]	Appendix II, section 1, subclause 1.12, first sentence

omit

5

insert

10

- [35]	Appendix II, section 2, subclause 2.18, Category B1

omit

30

insert

32

- [36]	Appendix III, Part 2, Theoretical element, item 3, Duration:

omit

paragraphs (a) to (e)

insert

(a)	times developed during the training needs analysis processes for the theoretical element will be for tuition hours only and exclude any breaks, examinations, revision, preparation and aircraft visit; and

(b)	for standardisation purposes, 1 tuition hour means 60 minutes of teaching; and

(c)	all course applications must be supported by a detailed training needs analysis.

- [37]	Appendix III, Part 4, A, paragraph (d)

omit

The

insert

Subject to paragraph (j), the

- [38]	Appendix III, Part 4, A, paragraph (j)

omit

an excessive number

insert

more than 25%

- [39]	Appendix V and Note

substitute the following Appendix and Note

- Appendix V

- Recognised States

Nil

Note   New Zealand has a status under the Trans-Tasman Mutual Recognition Agreement that is equivalent to that of a recognised State.

- [40]	Appendix VI

substitute

- Appendix VI

- Excluded States

Nil

- [41]	Appendix VII

omit

all words after the Note

insert

E6	Excluding avionic LRUs

E11	Excluding audio CVR systems

E12	Excluding propellers

E13	Excluding hydraulics — ATA29

E14	Excluding vapour cycle air-conditioning aspects of ATA21

E15	Excluding air-conditioning aspects of ATA21

E16	Excluding pressurisation aspects of ATA21

E18	Excluding ADF systems

E19	Excluding VOR systems

E20	Excluding ILS systems

E21	Excluding weather radar systems

E22	Excluding ATC transponder systems

E23	Excluding radio altimeter systems

E24	Excluding DME systems

E25	Excluding doppler systems

E26	Excluding satellite navigation systems

E27	Excluding autopilots

E28	Excluding multi-axis autopilots

E29	Excluding remote indicating compass systems

E30	Excluding inertial navigation and reference systems

E31	Excluding pressurisation systems

E32	Excluding electrical systems in aircraft equipped with multi-generator powered systems

E33	Excluding supercharging

E34	Excluding digital systems

E35	Excluding pressurised structures

E36	Excluding carburettor systems

E37	Excluding fuel injection systems

E38	Excluding turbo supercharging systems

E39	Excluding airframe ice protection systems

E40	Excluding airframe fire protection systems

E41	Excluding oxygen systems

E42	Excluding landing gear retraction systems

E43	Excluding fabric other than flight controls

E44	Excluding wiring repairs