Fuel Standard (Ethanol E85) Determination 20121

Fuel Quality Standards Act 2000

I, Don Farrell, Parliamentary Secretary for Sustainability and Urban Water, make the following determination under section 21 of the Fuel Quality Standards Act 2000.

Dated 16th August	 2012

Don Farrell

Parliamentary Secretary for Sustainability and Urban Water

1	Name of determination

This determination is the Fuel Standard (Ethanol E85) Determination .

2	Commencement

This determination commences on 1 November 2012.

3	Definitions

In this determination:

ASTM followed by an alphanumeric code means the testing method developed under that alphanumeric code by the standards development organisation called ASTM International.

CEN/TS followed by a number means the testing method developed under that code and number by the standards development organisation called the European Committee for Standardization.

E85 means a fuel blend consisting of:

(a)	70–85% ethanol; and

(b)	petrol that meets the requirements of the Fuel Standard (Petrol) Determination 2001.

EN followed by a number means the testing method developed under that code and number by the standards development organisation called the European Committee for Standardization.

4	Fuel standard for E85

E85 must comply with the specification for the parameters mentioned in the following table.

| Item | Parameter | Specification |
| --- | --- | --- |
| 1 | Acidity (as acetic acid) | 0.006% m/m maximum |
| 2 | Benzene | 0.35% v/v maximum |
| 3 | Copper | 0.10 mg/kg maximum |
| 4 | Ethanol | 70–85% v/v |
| 5 | Ethers (5 or more C atoms) | 1.0% v/v maximum |
| 6 | Final boiling point (distillation) | 210°C maximum |
| 7 | Higher alcohols (C3-C8) | 2.0% v/v maximum |
| 8 | Inorganic chloride | 1 mg/kg maximum |
| 9 | Lead content | 5 mg/L maximum |
| 10 | Methanol | 0.5% v/v maximum |
| 11 | Motor Octane Number | 87 minimum |
| 12 | Oxidation stability | 360 minutes minimum |
| 13 | pHe | 6.5–9.0 |
| 14 | Phosphorus | 1.3 mg/L maximum |
| 15 | Research Octane Number | 100 minimum |
| 16 | Solvent washed gum | 5 mg/100mL maximum |
| 17 | Sulfate | 4.0 mg/kg maximum |
| 18 | Sulfur | 70 mg/kg maximum |
| 19 | Vapour Pressure (DVPE) | 38–65 kPa at 37.8°C |
| 20 | Water | 1.0% m/m maximum |

5	Testing methods

(1)	Compliance with the standard for a parameter set out in section 4 is determined by applying the testing method for the parameter in the following table.

| Item | Parameter | Testing method |
| --- | --- | --- |
| 1 | Acidity (as acetic acid) | ASTM D1613 |
| 2 | Benzene | ASTM D5580 |
| 3 | Copper | EN 15837 (as modified in CEN/TS 15293) |
| 4 | Ethanol | ASTM D5501 |
| 5 | Ethers (5 or more C atoms) | ASTM D4815 (See Note 1) |
| 6 | Final boiling point (distillation) | ASTM D86 |
| 7 | Higher alcohols (C3-C8) | ASTM D4815 (See Note 1) |
| 8 | Inorganic chloride | ASTM D7328 |
| 9 | Lead content | ASTM D3237 |
| 10 | Methanol | ASTM D5501 |
| 11 | Motor Octane Number | (See Note 2) |
| 12 | Oxidation stability | ASTM D525 |
| 13 | pHe | ASTM D6423 |
| 14 | Phosphorus | ASTM D3231 |
| 15 | Research Octane Number | (See Note 2) |
| 16 | Solvent washed gum | ASTM D381 |
| 17 | Sulfate | ASTM D7319 |
| 18 | Sulfur | ASTM D5453 |
| 19 | Vapour Pressure (DVPE) | ASTM D5191 |
| 20 | Water | ASTM E1064 |

Note 1   Testing method ASTM D4815–09 has been validated for use in the analysis of oxygenates (ethers (5 or more C atoms) and higher alcohols (C3-C8)) in ethanol/unleaded petrol blended fuels containing 70 to 85% ethanol, providing standards are prepared in 80% ethanol blended with unleaded petrol.

Note 2   Testing methods for Motor Octane Number and Research Octane Number in E85 are not yet available. The minimum targets specified in section 4 are interim targets until a testing method is available. These minimum targets allow for engine calibration.

(2)	A testing method mentioned in subsection (1) is one that is in force on the day on which this determination commences.

Note

1.	All legislative instruments and compilations are registered on the Federal Register of Legislative Instruments kept under the Legislative Instruments Act 2003. See www.comlaw.gov.au.