EXPLANATORY STATEMENT

Issued by the Australian Communications and Media Authority

Radiocommunications Advisory Guidelines (Managing Interference to Spectrum Licensed Receivers – 1800 MHz Band) 2012

Radiocommunications Act 1992

Purpose

The purpose of the Radiocommunications Advisory Guidelines (Managing Interference to Spectrum Licensed Receivers – 1800 MHz Band) 2012 (the Advisory Guidelines) is to provide information and guidance to assist with the management of interference to radiocommunications receivers operating under spectrum licences issued for the 1800 MHz band caused by radiocommunications transmitters operating under class licences, and from fixed transmitters operating under apparatus licences that are issued on or after 18 June 2013. The Advisory Guidelines are designed to be used by the holders of apparatus and spectrum licences in the planning of services or the resolution of interference.

Legislative provisions

Under section 262 of the Radiocommunications Act 1992 (the Act), the Australian Communications and Media Authority (the ACMA) may make advisory guidelines about any aspect of radiocommunication or radio emissions.

Subsection 262(2) of the Act provides a non-exhaustive list of examples of the matters about which advisory guidelines may be made, one of which is ‘interference with radiocommunications’.

The Advisory Guidelines are a legislative instrument under the Legislative Instruments Act 2003.

- Background

The 15 year spectrum licences in the 1710-1785 MHz and 1805-1880 MHz band (the 1800 MHz band) were issued in two tranches, the first in 1998 and the second in 2000.

A spectrum licence permits a licensee, subject to specified conditions, to operate radiocommunications devices within a particular spectrum space, defined by a frequency band and a geographic area. Interference occurring between adjacent spectrum licences consists of in-band interference across the geographic boundaries, and out-of-band interference across the frequency boundaries. Interference can also occur between spectrum licensed services and services operating under apparatus and class licensing arrangements respectively.

The Act provides a number of means by which the ACMA may manage interference that may be caused to a radiocommunications receiver operating under a spectrum licence. One of these includes the ability to make advisory guidelines under section 262 of the Act about interference.

The Radiocommunications Advisory Guidelines (Managing Interference from Apparatus-licensed and Class-licensed Transmitters – 1800 MHz Band) 1999 (the 1999 Guidelines) made under section 262 of the Act provide guidance to assist with the protection of spectrum licensed radiocommunications receivers operating in the 1800 MHz band from apparatus and class licensed radiocommunications transmitters. Similarly,the Radiocommunications Advisory Guidelines (Protection of Apparatus-licensed and Class-licensed Receivers – 1800 MHz Band) 1999 and the Radiocommunications Advisory Guidelines (Protection of Mobile Base Receivers – 1800 MHz Lower Band) 1999 made under section 262 of the Act deal with managing interference in other specific circumstances.

Current spectrum licences in the 1800 MHz band will expire either on 17 June 2013 (for licences issued in 1998) or on 3 May 2015 (for licences issued in 2000). To prepare for the re-issue and/or re-allocation of spectrum licences in the 1800 MHz band, the ACMA conducted a review of the 1800 MHz spectrum licensing technical framework. The aim of the review was to:

- ensure flexibility so that a range of modern technologies can be used in the band, with a particular focus on International Mobile Telecommunications (IMT) technologies;

- provide conditions that enable continued usage of existing network technologies in the band;

- provide interference management within the 1800 MHz band, and in adjacent bands; and

- address deficiencies that have come to light during the current licence period.

The review recommended that the 1999 Guidelines be amended to account for the modernisation of mobile communications technologies that has occurred since the publication of 1999 Guidelines, as well as to allow for developments that may be expected to take place in the next spectrum licence period.

The Advisory Guidelines are one of a set of legal instruments being made by the ACMA to vary the spectrum licensing technical framework applicable to the 1800 MHz band according to the review recommendations. The Advisory Guidelines revoke the 1999 Guidelines and implement the review recommendations.  The ACMA will also make the Radiocommunications Advisory Guidelines (Managing Interference from Spectrum Licensed Transmitters – 1800 MHz Band) 2012, Radiocommunications Advisory Guidelines (Additional Device Boundary Criteria – 1800 MHz Lower Band) 2012 and the Radiocommunications (Unacceptable Levels of Interference – 1800 MHz Band) Determination 2012.

Operation

The Advisory Guidelines provide guidance on the management and settlement of interference to radiocommunications receivers operating under spectrum licences in the 1800 MHz band which is caused by radiocommunications transmitters operating under class licences or apparatus licences in adjacent bands or adjacent areas.

The Advisory Guidelines should be used by holders of spectrum and apparatus licences in the planning of services or the resolution of interference. The ACMA also takes the Advisory Guidelines into account when determining whether a spectrum licensee is causing interference to a licensed radiocommunications receiver that is operating in accordance with its licence conditions.

- Consultation

The ACMA has consulted extensively with stakeholders about the review of the spectrum licensing technical framework for the 1800 MHz band.

In July 2011, the ACMA established an advisory body known as a Technical Liaison Group (TLG) to support the review of the technical framework in the 1800 MHz band. Incumbent and prospective licensees for the 1800 MHz band were invited to participate in the TLG process.

The TLG was tasked to consider and provide advice to the ACMA on technical aspects required for the development or review of the technical framework for the 1800 MHz band. This included consideration of:

- the core conditions of the spectrum licence in accordance with section 66 of the Act;

- the unacceptable levels of interference determination made under subsection 145(4) of the Act for the 1800 MHz band;

- the radiocommunications advisory guidelines made under section 262 of the Act for the 1800 MHz band;

- the draft spectrum licence; and

- the minimum contiguous bandwidth for spectrum licences in the 1800 MHz band.

The ACMA developed three discussion papers which outlined the proposed approach to the spectrum licensing framework for the 1800 MHz band.  These papers were provided for comment by the ACMA to TLG members and are available on the ACMA website at http://www.acma.gov.au.

The ACMA took into account the views expressed by TLG members when preparing the draft Advisory Guidelines. The draft Advisory Guidelines were available for public comment from 27 June 2012 to 27 July 2012 in order to give all interested parties an opportunity to comment on the draft technical framework instruments before the final Advisory Guidelines were made by the ACMA.

There were 2 submissions received during public consultation regarding the revised technical framework. These submissions provided no comment regarding the draft Advisory Guidelines.

Regulatory impact

Prior to releasing the draft Advisory Guidelines, the ACMA consulted with the Office of Best Practice Regulation (the OBPR) on the requirement for a regulation impact statement (RIS) for this legislative instrument. The OBPR advised that the Advisory Guidelines do not warrant the preparation of a RIS because the instrument is likely to have only minor and machinery impacts. The reference number for the OBPR’s assessment is OBPR, ID 14048.

Documents incorporated by reference

The Advisory Guidelines incorporate the following document by reference:

- The Radiocommunications Advisory Guidelines (Additional Device Boundary Criteria – 1800 MHz Lower Band) 2012 which is a legislative instrument made by the ACMA under section 262 of the Act which describes the additional device boundary criteria to be considered before registering transmitters operating in the 1800 MHz band. Copies of this legislative instrument may be obtained from the Australian Government’s Comlaw website at http://www.comlaw.gov.au.

Detailed description of the instrument

Details of the instrument are set out in Attachment A.

Statement of compatibility with human rights

Subsection 9(1) of the Human Rights (Parliamentary Scrutiny) Act 2011 requires the rule maker in relation to a legislative instrument to which section 42 (disallowance) of the Legislative Instruments Act 2003 applies to cause a statement of compatibility to be prepared in respect of that legislative instrument. This statement is Attachment B.

ATTACHMENT A

DETAILS OF THE RADIOCOMMUNICATIONS ADVISORY GUIDELINES (MANAGING INTERFERENCE TO SPECTRUM LICENSED RECEIVERS – 1800 MHZ BAND) 2012

Section 1 – Name of Advisory Guidelines

This section provides that the name of the instrument is the Radiocommunications Advisory Guidelines (Managing Interference to Spectrum Licensed Receivers – 1800 MHz Band) 2012.

Section 2 - Commencement

This section provides that the Advisory Guidelines commence on 18 June 2013.

Section 3 – Revocation

This section revokes the Radiocommunications Advisory Guidelines (Managing Interference from Apparatus-Licensed and Class-licensed Transmitters – 1800 MHz Band) 1999.

Section 4 – Purpose of these Advisory Guidelines

This section states the purpose of the Advisory Guidelines is to assist in managing in-band and out-of-band interference to registered spectrum licensed radiocommunications receivers operating in the 1800 MHz band. The Advisory Guidelines define the circumstances under which protection will be afforded to receivers from radiocommunications transmitters operating under a class licence, and from fixed transmitters operating under:

(i)	an apparatus licence issued on or after 18 June 2013; or

(ii)	a spectrum licence where the transmitter is registered under Part 3.5 of the Act on or after 18 June 2013.

This section also indicates that the Advisory Guidelines should be referred to in the planning of services in the 1800 MHz band and the resolution of any interference dispute that may arise between spectrum licensees and any licensees in adjacent licence areas and bands.

Section 5 – Interpretation

This section provides definitions for the terms used in the Advisory Guidelines. The section also states that unless a contrary intention appears, terms used in the Advisory Guidelines have the same meaning as in the Radiocommunications (Unacceptable Levels of Interference – 1800 MHz Band) Determination 2012 or in the Act.

Part 1 - Background

This Part provides some basic information about spectrum licences and the modes of interference occurring across frequency boundaries and geographical boundaries.  It describes the components of the 1800 MHz technical framework that are used to manage interference, which consist of:

- core conditions applying to spectrum licences and made in accordance with section 66 of the Act;

- the determination on unacceptable levels of interference made in accordance with section 145 of the Act; and

- the advisory guidelines on managing interference to and from spectrum licensed services in specific circumstances made in accordance with section 262 of the Act.

It also noted that the Advisory Guidelines have been made to provide guidance in the resolution of cases of interference occurring to spectrum licensed radiocommunications receivers and caused by radiocommunications transmitters operated under other licences operating in the 1800 MHz band.

Part 2 - Managing interference from other services

Section 2.1 – In-band interference

This section explains how in-band interference to a radiocommunications receiver operating under a spectrum licence is managed.

In-band interference from a radiocommunications transmitter operating under an adjacent spectrum licence is managed through the core conditions imposed on spectrum licences, the device boundary criteria and deployment criteria prescribed in the section 145 determination on unacceptable levels of interference for the 1800 MHz band, and additional device boundary criteria defined in the Radiocommunications Advisory Guidelines (Additional Device Boundary Criteria – 1800 MHz Lower Band) 2012.

In-band interference from a radiocommunications transmitter operating under an apparatus licence is managed as if the transmitter is operated under a spectrum licence. The device boundary criteria applicable to spectrum licensed transmitters also apply to apparatus licensed transmitters which operate under licences issued on or after 18 June 2013.  As a result, new spectrum licences are afforded the same level of in-band protection from new apparatus licensed radiocommunications transmitters as they are from radiocommunications transmitters operated under adjacent spectrum licences.

It is also noted in this section that the Advisory Guidelines do not cover interference caused by transmitters operating under an apparatus licence that was issued before 18 June 2013.

This section also explains that the ACMA will not regard in-band interference to a radiocommunications receiver operating under a spectrum licence caused by a radiocommunications transmitter operating under a class licence as unacceptable if the operation of the transmitter complies with all relevant conditions of the class licence.

Section 2.2 – Out-of-band interference

This section sets out what constitutes out-of-band interference in a radiocommunications receiver operated under a spectrum-licence.  Out-of-band interference can occur under a number of circumstances as described in the Advisory Guidelines.  It may consist of intermodulation products, harmonic signals, parasitic signals or other spurious signals generated internally to the radiocommunications receiver.

Out-of-band interference may also extend for significant frequency separations on either side of a spectrum licence and its severity may depend on the quality of the radiocommunications receiver.  For these reasons, out-of-band interference is managed through the definition of a notional receiver performance level and a compatibility requirement for coordination with apparatus licensed services.  The use of a performance standard for spectrum licensed receivers ensures that the burden of mitigating interference is not solely placed on the transmitter side.

Section 2.3 - Recording radiocommunications receiver details in the Register

This section explains that in order for a spectrum licensed radiocommunications receiver to be afforded protection from interference under the Advisory Guidelines, details of the receiver must be recorded in the register of radiocommunications licences the Register.  Where interference is caused by a spectrum licensed radiocommunications transmitter, the details of the receiver must be included in the Register before the details of the transmitter are placed on the Register. Where interference is caused by an apparatus licensed radiocommunications transmitter, the details of the receiver must be included in the Register before the date of issue of the apparatus licence under which the transmitter operates.

Section 2.4 – Mobile and nomadic devices

This section specifies that the compatibility requirement (specified in Part 4 of the Advisory Guidelines) does not apply to mobile or nomadic devices due to their transient nature.

Part 3		Minimum level of receiver performance

Section 3.1 – Notional receiver performance

This section explains the basis for definition of the notional receiver performance level.  The degree of interference seen in a radiocommunications receiver is dependent on the nature of the emissions from a radiocommunications transmitter as well as the performance of the receiver. Emissions from transmitters should not have to be reduced below a point where the performance of a receiver is the problem.  The section also states that a registered radiocommunications receiver should meet the notional receiver performance level set out in Schedule 1 in order to obtain protection in accordance with the Advisory Guidelines.

This does not mean licensees are required to always adhere to the notional receiver performance level specified in Schedule 1. However, the compatibility requirement described in Part 4 of the Advisory Guidelines is based on a system that performs to this level or better. In order to gain the full benefits, a receiver should meet or exceed the notional receiver performance. Also when assessing interference, the ACMA will assume receivers perform to the levels specified in the Advisory Guidelines.

This section also contains a note stating the notional receiver performance described in Schedule 1 is anticipated receiver performance. The performance is based on information available to the ACMA at the time the Advisory Guidelines are made. The note states that the notional receiver performance level may be amended in the future, but any changes would only be introduced after consultation with holders of 1800 MHz band spectrum licences.

Part 4 – Compatibility requirement

Section 4.1 – Compatibility

This section explains what is required for applying the compatibility requirement for the protection of a spectrum licensed radiocommunications receiver from a radiocommunications transmitter operating under an apparatus or spectrum licence.  The licensee of a fixed transmitter operating under an apparatus licence or a spectrum licence is required to adhere to the compatibility requirement for those spectrum licensed receivers that:

- meet the notional receiver performance level, set out in Schedule 1;

- are registered in the Register before:

  - the transmitter is registered (if the transmitter is operated under a spectrum licence); or

  - the date of issue of the apparatus licence under which the transmitter operates; and

- operate at effective antenna heights below the limits for the 1800 MHz upper band and 1800 MHz lower band set out in section 4.1.

The compatibility requirement for transmitters operating under an apparatus or spectrum licence affords protection to receivers operated under a spectrum licence on a first-in-time basis. This means that in order to be afforded protection, the date of registration of the spectrum licensed receiver must be before the date on which a spectrum licensed transmitter is registered in the Register, or the date of issue of an apparatus licence if the transmitter is operating under an apparatus licence.

This section also provides what is required for applying the compatibility requirement for the protection of a spectrum licensed receiver from a transmitter operating under a class licence.  In this case a radiocommunications transmitter operating under a class licence must comply with the conditions of the class licence. If the conditions of the class licence are met then the class licensed device is deemed to meet the compatibility requirement.

Schedule 1 – Notional receiver performance level

This Schedule defines the notional receiver performance level for spectrum licensed radiocommunications receivers.  Such receivers should meet or exceed this performance level in order to minimise interference from spectrum licensed and apparatus licensed transmitters.  The notional receiver performance level consists of requirements for the following:

- Adjacent channel selectivity performance which is the measure of the ability of a radiocommunications receiver to receive a wanted signal without exceeding a specified degradation in output quality due to the presence of an unwanted adjacent channel signal.

- Intermodulation response rejection performance, which is defined as the measure of the ability of a radiocommunications receiver to receive a wanted signal in the presence of two or more unwanted signals with a specific amplitude and frequency relationship to the wanted signal frequency.

- Receiver blocking performance, which is the measure of the ability of a radiocommunications receiver to receive a wanted signal in the presence of a high level unwanted interferer on frequencies other than those of the adjacent channels.

- Receiver spurious response performance, which is a measure of the ability of the receiver to discriminate between the wanted signal and an unwanted signal at any frequency, outside the frequency band of the licence, to which the receiver responds.

The notional receiver performance level assumes:

- A standard receiver performance for modern mobile communications devices.

- The use of registered details on antenna gain and feeder losses in performing interference analysis. A representative combination of antenna gain and feeder/branching losses is provided in the event these are not provided.

- A radiofrequency selectivity, between the antenna and the antenna connector of the receiver at least equal to that specified in section (7) of the Schedule.

Schedule 2 – Compatibility requirement

This Schedule defines the compatibility requirement for managing interference from radiocommunications transmitters operating under an apparatus or spectrum licence.

ATTACHMENT B

Statement of Compatibility with Human Rights

Prepared in accordance with Part 3 of the Human Rights (Parliamentary Scrutiny) Act 2011

Radiocommunications Advisory Guidelines (Managing Interference to Spectrum Licensed Receivers – 1800 MHz Band) 2012

This legislative instrument is compatible with the human rights and freedoms recognised or declared in the international instruments listed in section 3 of the Human Rights (Parliamentary Scrutiny) Act 2011.

Overview of the Legislative Instrument

Section 262 of the Radiocommunications Act 1992 (the Act) permits the Australian Communications and Media Authority (the ACMA) to make advisory guidelines about any aspect of radiocommunication or radio emissions.

The purpose of the Radiocommunications Advisory Guidelines (Managing Interference to Spectrum-Licensed Receivers – 1800 MHz Band) 2012 (the Advisory Guidelines) is to provide information and assist in the protection of radiocommunications receivers operating under spectrum licences issued for the 1800 MHz band from interference caused by radiocommunications transmitters operating under class licences, spectrum licences, or apparatus licences that are issued on or after 18 June 2013.

Subsection 9(1) of the Human Rights (Parliamentary Scrutiny) Act 2011 requires the rule-maker in relation to a legislative instrument to which section 42 (disallowance) of the Legislative Instruments Act 2003 applies to cause a statement of compatibility to be prepared in respect of that legislative instrument.

The Advisory Guidelines are a legislative instrument subject to disallowance under section 42 of the Legislative Instruments Act 2003.

Human Rights Implications

The Advisory Guidelines do not engage any of the applicable rights or freedoms.

Conclusion

The Advisory Guidelines are compatible with human rights as they do not raise any human rights issues.